SONIC SCULPTURE II

Technical Architecture & Prototype Planning

Did Sun Ra Fear Cryosleep?

Darren Woodland Jr. | PhD Dissertation Research

Speculocultural Technopoiesis — Research-Creation

March 2026 | v0.1 Draft

# 1. Conceptual Frame

Did Sun Ra Fear Cryosleep? is the second movement of a three-part dissertation research-creation triptych exploring Black sonic epistemologies as computational logic. Where Movement I (Buffalo Soldier) investigates spatial sonicity and Movement III (Don't Play with My Hair) engages bodily-haptic sonic politics, Movement II operates in the register of temporal sonicity — the suspension, dissolution, and slow transformation of matter and memory across time.

The piece takes as its material the acoustic micro-events of physical dissolution: sugar, salt, clay, and effervescent compounds dissolving in water, captured via DIY hydrophones built from piezoelectric disks with JFET impedance-matching preamp circuits. These dissolution sounds are processed in real-time through granular synthesis — a technique that atomizes sound into grain-clouds and reassembles them, enacting in the audio domain what cryosleep implies for the body: suspension at the threshold of transformation, neither fully present nor fully dissolved.

Methodological note: The choice of hardware platform, sensing apparatus, and signal chain is not merely technical — it is itself a site of Speculocultural Technopoiesis. The modifications made to tools (hydrophone circuits, sensor fusion, RNBO patch compilation to embedded hardware) enact Black sonic epistemologies at the level of computational logic, not merely as content but as method.

# 2. Platform Decision Rationale

Platform selection was informed by Di Maggio, Hengeveld & Tanaka's "Embedded Comparo: Small DSP Systems Side-by-Side" (NIME 2025), which provides a comparative analysis of Bela, Daisy, OWL, and Raspberry Pi for embedded musical instrument deployment. Key findings directly relevant to this work:

| Platform | Type | RNBO Support | Granular (Cloud) | Latency | Verdict |
| --- | --- | --- | --- | --- | --- |
| Bela / Bela Gem | SBC (BeagleBone) | YES (official) | YES — ran successfully | Sub-ms | Primary audio target |
| Daisy Seed | MCU (STM32H750) | Partial (minimal export) | FAILED (hvcc limits) | Low | Alternate / simpler patches |
| OWL / Lich | MCU (ARM M7) | No | FAILED (1MB memory) | Low | Eurorack only |
| Raspberry Pi | SBC (ARM Linux) | YES (official) | YES — full streaming | Buffer-dependent | Final install brain |

The NIME 2025 paper specifically notes that Atau Tanaka (co-author, Goldsmiths) has performed live with a Bela Pepper + cloud granular synthesis system in Brussels, London, Newcastle, Bristol, and Paris (2024–2025) — providing peer-validated proof of concept for the exact architecture proposed here. For the final installation, Bela Gem Stereo (launched October 2025) offers the same capabilities in a compact, embeddable form factor.

For the conference demo prototype, a laptop-based architecture reduces cost and setup complexity while preserving the same software stack (Max/RNBO, Plugdata, vvvv). The ESP32-based sensor node communicates with the laptop via WiFi OSC or USB serial, keeping the sensor/actuation layer identical between prototype and final installation.

# 3. Signal Chain Architectures

## 3.1  Conference Demo (Laptop)

The conference demo runs entirely from a laptop. The sensor node (ESP32 + Grove Shield) operates independently, transmitting sensor data wirelessly. The existing Steinberg UR22C or Zoom H5 Studio serves as the audio interface for hydrophone input and speaker output.

AUDIO STREAM ──────────────────────────────────────────────────── Piezo hydrophone disk(s) + JFET preamp ↓ analog audio (XLR / TRS) Steinberg UR22C OR Zoom H5 Studio (USB audio interface) ↓ USB Laptop — Max/RNBO (granular cloud synthesis / dissolution processing) ↓ USB audio out UR22C / H5 → portable amp → speakers SENSOR STREAM ──────────────────────────────────────────────────── Grove TDS sensor (A2 / ADC1 channel) ┐ Grove Turbidity sensor (A3 / ADC1 channel) ├─ Grove Shield FeatherWing Grove Thermistor (A4 / ADC1 channel) │ MOSFET actuation circuit (Digital 1) ┘ ↓ Adafruit ESP32 Feather V2 (WiFi OSC or USB-C serial fallback) ↓ Laptop — Max/RNBO receives OSC → modulates granular parameters Laptop — vvvv receives OSC → drives visual output VIDEO ──────────────────────────────────────────────────── Laptop HDMI → display / projector (vvvv visual output)

## 3.2  Final Installation (Standalone)

The final installation replaces the laptop with a Raspberry Pi 5 (8GB RAM) as the central processing brain, with a Bela Gem Stereo embedded inside the sculpture object handling ultra-low-latency audio. A Node.js communication hub on the Pi routes sensor data from the ESP32 to both the audio and visual processing layers. The Bela Gem Stereo runs a compiled RNBO or Pure Data patch standalone.

AUDIO STREAM ──────────────────────────────────────────────────── Piezo hydrophone(s) + JFET preamp ↓ analog audio Bela Gem Stereo (embedded in sculpture housing) ↓ runs RNBO / PD patch standalone (sub-ms latency) ↓ audio out Embedded amplifier → speakers (in / near sculpture) SENSOR STREAM ──────────────────────────────────────────────────── Grove sensors + MOSFET actuation ↓ ESP32 Feather V2 [LiPo battery powered, wireless] ↓ WiFi OSC Raspberry Pi 5 8GB — Node.js hub ↓ OSC → Bela Gem (audio param modulation via serial/OSC) ↓ OSC → vvvv / visual patcher (generative visuals) VIDEO ──────────────────────────────────────────────────── Pi 5 micro-HDMI → display / projector

# 4. Sensing Layer

## 4.1  Hydrophone (Primary Audio Sensor)

DIY hydrophones built from piezoelectric disk elements. The piezo disk is a broadband pressure sensor that responds to acoustic micro-events in liquid — the pops, fizzes, structural collapses, and vibrational signatures of materials losing cohesion. Critical to signal quality is impedance matching: raw piezo disks present ~10 MΩ impedance and without a buffer stage, low frequencies are lost and noise dominates.

| Parameter | Spec / Notes |
| --- | --- |
| Transducer element | Piezoelectric disk (27mm typical) |
| Preamp circuit | JFET buffer — J201 or 2N5457 recommended |
| Function of JFET | Impedance matching (10MΩ → ~1kΩ); preserves low-frequency response |
| Output | Mono audio line/mic level — XLR or 1/4" TRS to audio interface |
| Test variable | MOSFET param in Dissolution Test JSON tracks preamp presence (0 = direct, 1 = buffered) |
| Materials tested | Sugar (tap/plastic), Seltzer/effervescence (tap/plastic) |
| Planned materials | Salt, clay, pigment, milk — varying optical/electrical dissolution profiles |

## 4.2  Grove Sensor Suite (Dissolution State)

Three Grove sensors capture the physical state of dissolution across electrical, optical, and thermal dimensions. Together they form a multi-parameter envelope of the dissolution event that modulates granular synthesis parameters in real time. All sensors connect via the Grove Shield FeatherWing on the ESP32 Feather V2.

| Sensor | Grove Product | Measures | Pin | Maps to (audio) |
| --- | --- | --- | --- | --- |
| TDS / Conductivity | Grove TDS Sensor (101020753) | Total Dissolved Solids (ppm) — rises as material dissolves | A2 (ADC1 ch6) | Granular grain density / cloud density |
| Turbidity | Grove Turbidity Sensor v1.0 | Optical clarity of water — changes with suspended particles | A3 (ADC1 ch3) | Playback position / time-stretch rate |
| Temperature | Grove Waterproof Thermistor | Water temperature — dissolution is often endo/exothermic | A4 (ADC1 ch0) | Transposition / pitch shift |

ADC note: ESP32 ADC2 channels (including A0/GPIO26 and A1/GPIO25 on Feather layout) are unreliable when WiFi is active. Use ADC1 channels exclusively for Grove analog sensors (A2=GPIO34, A3=GPIO39, A4=GPIO36). For higher precision, an ADS1115 16-bit ADC breakout (I2C, via Grove I2C / STEMMA QT port) can replace direct ADC reads.

## 4.3  Actuation (Dissolution Triggering)

Dissolution in water is not passive — flow, temperature, vibration, and acoustic energy all accelerate the process. The sculpture's actuation layer introduces controlled dissolution events, creating a feedback loop between the sensing and the sounding. The conceptual resonance is explicit: in oceans and rivers, dissolution is assisted by physical forces; in the Afrofuturist imaginary of cryosleep and deep space, dissolution-as-transformation operates under entirely different physics.

| Actuator | Mechanism | Driver | Notes |
| --- | --- | --- | --- |
| Ultrasonic transducer (40kHz piezo) | Acoustic cavitation — dramatically accelerates dissolution | Dedicated ultrasonic driver board (~$5–10) | Same piezo family as sensing element; the instrument that listens also dissolves |
| Small peristaltic pump | Water circulation — keeps dissolution active | MOSFET (N-channel, e.g. 2N7000) via ESP32 digital pin | Controls flow rate; also audible as low-frequency pulse |
| Solenoid valve (optional) | Controls introduction of new material into vessel | MOSFET low-side switch | Enables timed dissolution events during piece |

# 5. Microcontroller: ESP32 Feather V2 + Grove Shield FeatherWing

The sensor node is built on the Adafruit ESP32 Feather V2 (HUZZAH32) with the Grove Shield FeatherWing as an add-under. This combination provides plug-and-play access to the Grove sensor ecosystem without soldering, while the ESP32's WiFi enables wireless OSC transmission to the laptop or Pi 5.

| Component | Key Spec | Role in System |
| --- | --- | --- |
| Adafruit ESP32 Feather V2 | ESP32 Pico, 8MB Flash, 2MB PSRAM, WiFi/BT, STEMMA QT, USB-C | Sensor polling, OSC transmission, actuation control |
| Grove Shield FeatherWing | 3× analog, 2× digital, 2× I2C (STEMMA QT compatible), 1× UART | Zero-solder Grove sensor connectivity |
| LiPo battery (3000mAh) | 3.7V JST, onboard charging via USB-C | Wireless power for demo — no cable to laptop |
| ADS1115 breakout (optional) | 16-bit I2C ADC, 4 channels, via I2C Grove port | Higher precision analog reads; avoids ESP32 ADC noise |

## Grove Shield Pin Allocation

Grove Shield FeatherWing — Pin Allocation (ESP32 Feather V2) Analog 1 (A2 / GPIO34 / ADC1) → Grove TDS Sensor Analog 2 (A3 / GPIO39 / ADC1) → Grove Turbidity Sensor Analog 3 (A4 / GPIO36 / ADC1) → Grove Waterproof Thermistor I2C 1 (SDA/SCL via STEMMA QT) → ADS1115 (optional precision ADC) I2C 2 (SDA/SCL) → Reserved / additional I2C sensor Digital 1 → MOSFET gate (actuation: pump / solenoid) Digital 2 → Reserved (ultrasonic driver trigger) UART → Wired serial fallback to laptop/Pi if needed

# 6. Software Stack

| Layer | Software | Platform | Function |
| --- | --- | --- | --- |
| Sensor firmware | Arduino / PlatformIO (C++) | ESP32 Feather V2 | Poll Grove sensors, control actuators, transmit OSC |
| Audio processing | Max/MSP + RNBO | Laptop / Pi 5 | Granular cloud synthesis; receives OSC params from sensor node |
| Audio processing | Plugdata (Pure Data) | Laptop / Pi 5 / Bela | Alternative PD-based granular; direct Daisy / Bela export |
| Visual processing | vvvv gamma | Laptop / Pi 5 | Generative visuals driven by sensor + audio analysis data |
| Comm hub | Node.js + node-osc | Pi 5 (final install) | Routes sensor OSC to audio + visual layers; logs data; web dashboard |
| Embedded audio | RNBO export / PD (hvcc) | Bela Gem Stereo | Standalone granular synthesis in final sculpture object |

### RNBO Pathway Note

Max/RNBO supports direct export to both Bela (official, via C++ / browser IDE) and Raspberry Pi (official export target). Export to Daisy Seed uses a "Minimal Export" C++ pathway (community-supported, Cycling '74 GitHub repo available). Per the NIME 2025 paper, RNBO is listed as supported on Bela and Raspberry Pi; Daisy support should be verified against current Cycling '74 documentation before committing to that target for complex granular patches, as hvcc limitations affect MCU-based deployment.

# 7. Power Architecture

## 7.1  Conference Demo

| Component | Power Source | Notes |
| --- | --- | --- |
| Laptop | Laptop charger | Powers Max/RNBO, vvvv, receives USB audio + USB serial |
| UR22C / Zoom H5 | USB bus power from laptop | ~500mA; laptop USB port sufficient |
| ESP32 Feather V2 | 3000mAh LiPo (JST onboard) | Fully wireless sensor node; charge overnight via USB-C |
| Actuation circuit | 5V/2A USB adapter (separate) | Isolates inductive load transients from sensor supply |
| Portable amp/speakers | Own power supply / battery | Per existing hardware |

## 7.2  Final Installation (Pi 5)

| Component | Power Source | Notes |
| --- | --- | --- |
| Raspberry Pi 5 8GB | Official Pi 5 USB-C PSU (5V/5A) | Non-negotiable; generic chargers cause instability |
| USB Audio Interface | Pi 5 USB bus (500mA) | Bus-powered from Pi USB port |
| Bela Gem Stereo | Dedicated 5V/2A supply OR USB | Check Bela Gem power requirements; may want isolated supply |
| ESP32 Feather V2 | 3000mAh LiPo | Wireless; charged between exhibitions |
| Actuators | Dedicated 5V or 12V adapter | Separate from Pi supply; select voltage per actuator spec |

# 8. Bill of Materials — Conference Demo Prototype

Items already owned are noted. This list covers what needs to be acquired to build a functional conference poster demo.

| Item | Source | Est. Cost | Status |
| --- | --- | --- | --- |
| Adafruit ESP32 Feather V2 (HUZZAH32) | Adafruit (#5400) | $19.95 | To purchase |
| Grove Shield FeatherWing | Adafruit / Seeed | $7.95 | To purchase |
| Grove TDS Sensor (101020753) | Seeed Studio | $9.90 | To purchase |
| Grove Turbidity Sensor v1.0 | Seeed Studio | $9.90 | To purchase |
| Grove Waterproof Temperature Sensor | Seeed Studio | $5.90 | To purchase |
| LiPo Battery 3.7V 3000mAh (JST-PH) | Adafruit / Amazon | $14.95 | To purchase |
| Grove cables (4-pin, various lengths) | Seeed Studio / Amazon | $5–10 | To purchase |
| Ultrasonic transducer 40kHz + driver board | Amazon / AliExpress | $8–12 | To purchase |
| N-channel MOSFET (2N7000 or similar, ×5) | DigiKey / Mouser | $2–3 | To purchase |
| Small peristaltic pump (5V) | Amazon / AliExpress | $10–15 | To purchase |
| Piezo disk elements 27mm (×10) | Amazon / Mouser | $5–8 | Likely have some |
| JFET transistors J201 or 2N5457 (×10) | Mouser / DigiKey | $3–5 | To purchase |
| Weatherproof/waterproof epoxy (for sealing) | Hardware store | $8–12 | To purchase |
| Laptop (Mac/PC) | — | Owned | Have |
| Steinberg UR22C / Zoom H5 Studio | — | Owned | Have |
| Portable amp + speakers | — | Owned | Have |
| USB-C power adapter for ESP32 charging | — | Owned | Have |

Estimated total for new purchases: ~$110–$140 USD

# 9. Additional Hardware — Final Installation

Items to acquire after the conference demo, for the final installation build.

| Item | Source | Est. Cost | Notes |
| --- | --- | --- | --- |
| Raspberry Pi 5 (8GB RAM) | Approved Pi 5 reseller | $80 | Central processing brain |
| Official Pi 5 USB-C PSU (5V/5A) | Raspberry Pi / Adafruit | $12 | Non-negotiable stability |
| Bela Gem Stereo | Bela (bela.io) | $280–320 | Embedded audio, sub-ms latency; shipping October 2025+ |
| HiFiBerry DAC2 Pro HAT (alt.) | HiFiBerry | $45 | Lower cost audio out for Pi if Bela Gem overkill for visuals layer |
| USB-C 5A PSU (Pi 5 dedicated) | — | $12 | Separate from Bela power |
| Micro-HDMI to HDMI cable | Amazon | $8 | Pi 5 video output |
| Pi 5 heatsink / case | Adafruit / Amazon | $10–20 | Thermal management for embedded install |

# 10. Development Roadmap

| Phase | Focus | Target |
| --- | --- | --- |
| Phase 0 — Cardboard mockup | Physical form exploration; sensor placement; vessel geometry; no electronics required | Immediately / ongoing |
| Phase 1 — Hydrophone build | DIY piezo + JFET preamp circuits; dissolution recording sessions (sugar, salt, clay, seltzer); document signal chain variations | As hardware arrives |
| Phase 2 — Sensor node | ESP32 + Grove Shield assembly; firmware for sensor polling + OSC transmission; calibration of TDS/turbidity/temp readings | As hardware arrives |
| Phase 3 — Max/RNBO patch | Granular cloud synthesis patch receiving OSC sensor params; map TDS → grain density, turbidity → playback position, temp → transposition | Parallel with Phase 2 |
| Phase 4 — Conference demo | Integration test: full laptop-based chain; cardboard mockup with live dissolution vessel; vvvv visual layer; poster demo rehearsal | Pre-conference |
| Phase 5 — Final install | Pi 5 + Bela Gem Stereo; Node.js hub; RNBO export to Bela; sculpture object build (non-cardboard); exhibition deployment | Post-conference |

# 11. Open Questions & Next Decisions

Vessel design. What is the material, form, and volume of the dissolution vessel embedded in or adjacent to the sculpture? This shapes sensor placement, actuator integration, and the acoustic behavior of the space.

vvvv on Pi 5. vvvv gamma ARM Linux support is active but should be tested before committing. Fallback: p5.js in Chromium browser (fully Pi-compatible) for conference demo visuals; vvvv on full machine for final install.

Conference visual scope. For the poster demo, what is the visual output — a secondary display showing generative visuals, or is the primary focus the sonic/interactive dissolution experience? Scoping this determines whether vvvv is needed immediately.

Dissolution materials. Which materials will be used for the conference demo vs. final piece? Different materials (salt, sugar, clay, milk, pigment) have very different acoustic, optical, and electrical dissolution signatures and should be tested systematically.

RNBO vs. Plugdata. Both are viable paths to Bela. RNBO stays within the Max environment; Plugdata (Pure Data front-end with direct Daisy/Bela export via hvcc) may offer a more stable compilation path for complex granular patches on MCU targets. Test both.

Actuation visibility. Is the actuation mechanism (pump, solenoid, ultrasonic driver) visually present in the sculpture, concealed, or aesthetically integrated? This is both a design and a methodological question — what is the piece showing about the process of dissolution?